RPS Fan Coil Cleaning Expectations

- Facility Operations Preventive Maintenance supervisors will notify building managers and RPS leadership of schedule and of any changes to set schedule ASAP

  - Make certain all material is on site prior to the start

- Communicate assignments and responsibilities with team members

  - Make certain to knock on each room door prior to entering

  - Communicate to each member their role

  - Follow ALL applicable safety procedures including LOTO

  - Make any needed notes

  - Certify check off sheets are completed each day

  - Ensure check off sheets are signed by the assigned FO staff member at the end of each shift

- Scan each completed check off sheet and attach to the appropriate work order number for documentation

  - Make sure all boxes are checked as appropriate

  - Make any needed tickets for the proper shops

- Check completed fan coil units and rooms

  - Inspect outside vendor’s work, is it what we asked them to do?  Does their work meet our standards

- Make certain all rooms entered have been secured

  - Make certain any related cleaning debris has been removed

- Do not alter or discard any standard operating procedure, guideline or policy without proper approval

  - Any changes to the job scope or recommendations by the vendor needs approved before implementation

Scope of Work

- Contractor Responsibility

  - Stage all equipment

  - Work with Indiana University RPS personnel to enter designated room

  - Remove grill from Fan Coil Unit

  - Remove dirty filter from unit

  - Remove inner panel from Fan Coil Unit

  - Sweep inside Fan Coil Unit removing all foreign material

  - Chemically clean coil and rinse with water

  - Install inner panel on Fan Coil Unit

  - New Filter provided by IU Facility Operations will be on site in designated areas

  - Clean front cover and grill removing all foreign material

  - Install front cover and grill

  - Clean grills above unit

- Additional Notes

  - Any furniture that must be moved for Fan Coil Unit access will be moved by RPS personnel

  - RPS personnel will be responsible for opening ALL student room doors to allow contractor access

  - RPS personnel will be responsible for making certain all student room doors are locked once all work has been completed

  - Any additional debris around the Fan Coil Unit created by the cleaning process will be the responsibility of the contractor to clean

  - Vendor will remove dirty filters from the premise and dispose of properly

  - Contractor will not be responsible for any Fan Coil Units that are only able to be accessed by ladder

  - Units will be randomly spot checked by Indiana University personnel for accurate work

- Pricing and Number of Units

  - Pricing will be provided as a "per unit" basis and not by the hour

  - Pricing will include all labor and chemicals

  - On campus, we are required to use Evap-Fresh coil cleaner as authorized by Environmental Health and Safety. If the contractor needs contact information for purchase, it will be provided

  - The number of units provided are an estimate. Final invoice will be based on the actual number of units completed

  - Numbers will vary due to room access, construction projects and could be less than the number estimated

  - It is the responsibility of the contractor to provide documented proof of number of units completed and will be subject to inspection

  - Invoices will be provided for each building completed and will reference the Purchase Order number, Work Order number and Building name as provided on PO

- Cleaning Dates

  - July 21st - Aug 8th

  - Dec. 22nd - Jan 9th

  - May 11th – May 29th

Note: Mandatory Pre-Bid Meeting to take place onsite at Indiana University Bloomington Campus

All participants MUST attend this site visit; failure to do so may result in immediate rejection of any proposal.

Meeting Date: Monday, 3/24 at 12:30pm EST

Meeting Location: Main West entry of Tulip Tree Apartments (2451 E 10th St, Bloomington, IN 47408)

From there we will visit Eigenmann and Forest. Teams calendar invite to follow for Pre-Bid Meeting.